### Instructions for Using the WinEMG Viewer

- Open the Application

  - Locate the ‘WinEMG’ folder and open it.

  - Double-click the ‘WinEMG_chanview_020725.mlapp’ file to launch the app.

- Set Up the MATLAB Path

  - Before using the app, ensure the ‘WinEMG’ folder and ‘helper functions’ subfolder are added to the MATLAB path.

- Load Data

OpenEphys Data

  - In the app, click the blue ‘Load OpenEphys Data’ button.

  - A file navigation window will appear. Navigate to the OpenEphys subfolder that contains the .oebin file (commonly named ‘recording1’).

    - Example path: Desktop/experiment15/recording1

    - In the MATLAB command window, you will be displayed the start and end time of the trial and will be prompted to enter a specific start time and end time that you wish to visualize. This speeds up plotting for long trials.

    - Once the data loads successfully, the command window will display: “Data successfully loaded for the specified time range”.

Intan Data

  - In the app, click the yellow ‘Load Intan Data’ button

  - A file navigation window will appear. Navigate to and select the specific Intan file (.rhd)

  - Once the data loads successfully, the command window will display: “Data successfully loaded”.

Custom Matlab file (Matt Williams)

- In the app, click the purple ‘Matt’s Large Matfile’ button

- A file navigation window will appear. Navigate to and select the specific matfile (.mat)

- You will be prompted to enter the sampling rate, start time and end time you wish to view.

- Once the data loads successfully, the command window will display: “Data successfully loaded for the specified time range”

- Configure Plot Parameters

  - Adjust the following parameters as needed:

    - Low and high cutoff frequencies

    - Selected channels to view

    - Channel offset

    - Filter smoothing (optional)

    - Start and end times (optional)

- Generate and Update Plots

  - Click the green ‘Generate Plot’ button to visualize the data.

  - To update the plot after modifying parameters, click ‘Generate Plot’ again.